# GCSE Design & Technology – Unit 3 Detailed Revision Guide by Topic

# Topic 1 – Energy Generation

Types of Energy Sources

- Non-renewable sources include coal, oil, and natural gas. These are finite and will eventually run out.

- Renewable sources include:

  - Solar energy – harnessed using photovoltaic cells to generate electricity.

  - Wind energy – captured using wind turbines; depends on wind strength and location.

  - Hydroelectric power (HEP) – uses the movement of water to turn turbines and produce electricity.

  - Tidal power – generated by the rise and fall of tides; reliable but site-specific.

  - Geothermal energy – harnesses heat from beneath the Earth’s surface; only available in certain locations.

  - Biomass – organic material burned to produce energy; considered renewable if replaced.

🔹 Key Advantages and Disadvantages

- Renewable energy sources are sustainable and have low environmental impact, but some depend on weather conditions.

- Non-renewable sources are reliable and currently widely used, but release carbon emissions and contribute to climate change.

🔹 Energy Generation in Practice

- The choice of energy depends on:

  - Environmental impact (e.g. emissions, habitat disruption).

  - Initial cost vs. long-term savings.

  - Availability of resources in the local area.

  - Suitability for the intended purpose or product (e.g. portability, scale).

🔹 Applications in Design & Technology

- Designers must consider the source of electricity powering their product.

- Products can be designed to be energy efficient or use renewable sources (e.g. solar-powered lights).

- Ethical designers aim to reduce reliance on fossil fuels by selecting alternative power sources.

Non-renewable energy sources include coal, oil, and gas – they will run out and release greenhouse gases.

Renewable energy sources include solar, wind, hydroelectric, tidal, biomass, and geothermal – sustainable and better for the environment.

Solar power uses photovoltaic cells to convert sunlight into electricity.

Wind turbines convert kinetic energy from wind into electricity – best in open, windy areas.

Hydroelectric power uses water flow to turn turbines and generate electricity.

Tidal energy uses the movement of tides – reliable but only suitable in specific locations.

Biomass is organic material burned or converted to produce energy – can be renewable if managed properly.

Geothermal energy uses heat from beneath the Earth – only possible in certain areas.

Choice of energy source depends on cost, location, availability, and environmental impact.

# Topic 2 – Powering Systems

What is a Powering System?

- A powering system provides movement or energy to make a product work.

- These systems convert input energy into mechanical output.

🔹 Types of Powering Systems

🔸 Mechanical Systems

- Use levers, gears, pulleys and other mechanisms to create motion.

- Involve forces, motion, and energy transfer.

- Often hand-powered or driven by motors.

🔸 Electrical Systems

- Use batteries, mains electricity, or solar cells to power products.

- Include switches, resistors, LEDs, motors, and buzzers.

🔸 Pneumatic Systems

- Use compressed air to transfer energy and produce movement.

- Common in factory automation and tools.

🔸 Hydraulic Systems

- Use pressurised liquid (usually oil) to create force and motion.

- Found in car brakes, diggers, and lifting equipment.

🔹 Energy Inputs and Outputs

- Input: energy going into the system (e.g. turning a crank, electricity).

- Process: the mechanism that transfers or changes the energy (e.g. gears, circuits).

- Output: the result (e.g. movement, light, sound).

🔹 Efficiency Considerations

- Designers aim to:

  - Reduce energy loss (e.g. through heat or friction).

  - Make systems more efficient and reliable.

  - Choose suitable power sources for the product’s size, use, and environment.

🔹 Applications in Products

- Electric toothbrush: uses electrical input and mechanical output.

- Lifting arm: may use hydraulic or pneumatic power.

- Toys: powered by batteries or solar cells with simple electrical systems.

# Topic 3 – Modern and Smart Materials

Smart Materials

- Smart materials respond to external stimuli (like temperature, light, or pressure) and change in a controlled way.

- They are reversible – they return to their original state when the stimulus is removed.

🔸 Examples of Smart Materials

- Shape memory alloys (SMAs):

  - Return to a preset shape when heated.

  - Example: Nitinol used in glasses frames and stents.

- Thermochromic pigments:

  - Change colour with temperature changes.

  - Used in mood rings, baby spoons, and thermometers.

- Photochromic pigments:

  - Change colour in response to light (especially UV).

  - Used in sunglasses that darken in sunlight.

- Hydrogels:

  - Absorb and retain water; swell or shrink with temperature or pH.

  - Used in nappies, wound dressings, and plant gels.

- Piezoelectric materials:

  - Generate electricity when compressed or vibrated.

  - Used in guitar pickups, keyless car entry, and sensors.

- Electroluminescent wire:

  - Emits light when an electrical current is applied.

  - Used in costumes, interior car design, and advertising.

🔹 Modern Materials

- Developed through recent advancements in technology or manufacturing.

- Designed to improve performance, efficiency, or sustainability.

🔸 Examples of Modern Materials

- Carbon fibre reinforced plastic (CFRP):

  - Strong, lightweight, and rigid.

  - Used in sports equipment, aerospace, and automotive.

- Kevlar:

  - Very strong and heat resistant.

  - Used in body armour and helmets.

- Polymorph:

  - Thermoplastic that becomes mouldable at 60°C.

  - Reusable and ideal for prototyping.

- Precious metal clays (PMC):

  - Made from metal particles (silver, gold, etc.) in a clay binder.

  - Moulded by hand and hardened in a kiln.

  - Ideal for jewellery and detailed components.

🔹 Applications in Design

- Products made with smart or modern materials are often more:

  - Responsive to the environment.

  - Durable, lightweight, or strong.

  - Appealing to users due to interactive features.

# Topic 4 – Composite Materials

What Are Composites?

- A composite material is made by combining two or more materials to create a new material with enhanced properties.

- Each material keeps its original properties, but the final material is better for a specific purpose.

🔹 Main Components of Composites

- Matrix (binder): the material that holds everything together (usually a resin or glue).

- Reinforcement: the material that improves strength or another property (e.g. fibres or particles).

🔹 Examples of Composite Materials

🔸 Glass Reinforced Plastic (GRP)

- Glass fibres embedded in plastic resin.

- Lightweight, strong, and corrosion-resistant.

- Used in boat hulls, car body parts, and storage tanks.

🔸 Carbon Fibre Reinforced Plastic (CFRP)

- Carbon fibres in a polymer resin.

- Very strong and lightweight but expensive.

- Used in bike frames, tennis rackets, aerospace, and F1 cars.

🔸 Concrete

- Cement, sand, and aggregate (stones) mixed with water.

- Strong in compression but weak in tension.

- Often reinforced with steel bars to increase tensile strength (called reinforced concrete).

🔸 Plywood

- Made from layers of wood veneer glued together with grain direction alternating for strength.

- Durable, stable, and resistant to splitting.

- Commonly used in furniture, flooring, and construction.

🔸 Kevlar composites

- Kevlar fibres combined with resin for lightweight, high-strength materials.

- Used in bulletproof vests, helmets, and protective gear.

🔹 Advantages of Composite Materials

- Can be stronger, lighter, and more durable than individual materials.

- Can be tailored for specific applications (e.g. high strength, flexibility, water resistance).

- Corrosion-resistant and suitable for extreme environments.

🔹 Disadvantages

- Often expensive and difficult to recycle.

- Require specialist manufacturing processes.

# Topic 5 – Technical Textiles

What Are Technical Textiles?

- Technical textiles are materials designed to perform specific functions rather than just look good.

- They often have enhanced performance, such as being stronger, lighter, heat-resistant, or durable.

🔹 Key Types of Technical Textiles

🔸 Agricultural textiles

- Used in agriculture and horticulture.

- Includes crop covers, fleece, and shade nets.

- Designed to protect crops from pests, weather, and disease.

🔸 Construction Textiles

- Used in construction.

- Includes materials like membranes, insulation, roofing layers, and concrete reinforcement textiles.

- Provide strength, flexibility, and weather resistance.

🔸 Medical textiles

- Used in medical and hygiene products.

- Examples: bandages, surgical gowns, wound dressings, and prosthetics.

- Must be sterile, absorbent, and non-toxic.

🔸 Geo textiles

- Used in civil engineering and groundworks.

- Help with soil stability, drainage, and erosion control.

- Found in road construction and embankments.

🔸 Protective Textiles

- Designed for personal protection.

- Includes bulletproof vests, fire-resistant suits, gloves, and helmets.

- Must be lightweight, durable, and resistant to heat or impact.

🔸 Sport textiles

- Used in sportswear and equipment.

- Examples: breathable fabrics, moisture-wicking materials, and padding.

- Must provide comfort, flexibility, and durability.

🔹 Smart Technical Textiles

- Some technical textiles use smart materials for enhanced performance.

- Example: Fabrics with thermochromic dyes that change colour with temperature.

- Embedded electronics can monitor body data (e.g. heart rate, GPS tracking in sportswear).

🔹 Environmental and Ethical Considerations

- Increasing use of recyclable fibres and sustainable materials.

- Designers must consider the environmental impact and end-of-life disposal of textiles.

# Topic 6 –

Topic 6 – Mechanical Devices

🔹 What is a Mechanical Device? • A mechanical device is a machine or tool with at least one moving part.
• Examples:

- Hammer: Not mechanical (no moving parts, though the body provides movement).

- Scissors: Mechanical (two blades = moving parts, fulcrum = pivot point).

🔹 Types of Motion

🔸 Linear Motion • Movement in a straight line in one direction.
• Examples: Sprinter, drag racing car.

🔸 Reciprocating Motion • Back-and-forth motion in a straight line.
• Examples: Jigsaw, hedge trimmer, pistons.

🔸 Rotary Motion • Circular motion.
• Examples: Drill bit, circular saw, clock hands.

🔸 Oscillating Motion • Back-and-forth movement on a curved path.
• Examples: Swing, monitoring waveforms (e.g. ECGs), Newton’s cradle.

🔹 Levers

• Levers include load, effort, and fulcrum.
• Examples: Tweezers, garlic press, seesaw.

🔸 Types of Levers              REMEMBER 			123

PLE

- First Order: Load and effort on opposite sides of the fulcrum.

  - Moving fulcrum closer to the load = less effort needed.

- Second Order: Load between effort and fulcrum.

  - Example: Wheelbarrow.

- Third Order: Effort between load and fulcrum.

  - Always has mechanical advantage < 1 (load moves further than effort).

🔹 Mechanical Advantage (MA) & Velocity Ratio (VR)

• MA = Load ÷ Effort
• VR = Distance moved by effort ÷ Distance moved by load

🔸 Efficiency • Real machines lose energy due to friction and are not 100% efficient.
• Efficiency (%) = (MA ÷ VR) × 100
• A rusty wheelbarrow is a good example of low efficiency due to friction.

🔹 Equilibrium

• Occurs when load and effort are equal and equidistant from the fulcrum (1:1 ratio).
• If one side is twice the mass, it must be half the distance from the fulcrum.

🔹 Linkages

• Used to change motion or force direction.

🔸 Bell Crank Linkage • Changes direction by 90°.
• Example: Bicycle brakes.

🔸 Reverse Motion Linkage • Input and output move in opposite directions.
• Example: Pop-up book mechanisms.

🔸 Crank and Slider • Converts rotary to reciprocating motion (or vice versa).
• Example: Engine pistons, steam locomotives.

🔹 Cams and Followers

🔸 Cam Types • Eccentric (circular): Steady rise and fall.
• Pear cam: Sudden rise/fall with long dwell (pause).
• Snail (drop) cam: Slow rise, sudden drop, works in one direction.

🔸 Follower Types • Flat: High friction, strong under load.
• Knife edge: Accurate, low friction, wears easily.
• Roller: Smooth, accurate, low friction, good under load.

🔹 Pulleys and Belts

• Pulleys change force direction or speed.
• Belts are made from rope, cable, canvas, rubber.
• Used in lifting gear, pillar drills, lathes.

🔹 Gears and Gear Trains

🔸 Gear Ratio
• Gear Ratio = Teeth on driven gear ÷ Teeth on drive gear

🔸 Gear Train • Only drive and driven gears matter for ratio.
• Idler gear changes direction but not the ratio.

🔸 Compound Gears • Found in gearboxes. Allow speed changes (slow/fast output).
• Used for pulling heavy loads (low gear) or speeding up (high gear).

🔸 Bevel Gears • Change motion direction by 90°.
• Used in hand drills, mixers, whisks.

🔸 Rack and Pinion • Converts rotary to linear motion.
• Used in steering systems, pillar drills.

🔹 Velocity Ratio (VR) in Pulleys and Gears

• For pulleys:
VR = Diameter of driven ÷ Diameter of driver
• For gears:
VR = Number of teeth on driven ÷ Number on driver

🔹 Input & Output Speeds (RPM)

• Output Speed (RPM) = Input Speed × Gear Ratio

# Topic 7 – Electrical Circuits

🔹 What is an Electronic System?

- A system is a group of parts that work together to perform a function.

- An electronic system uses electrical components to control inputs, processes, and outputs.

- Systems can be broken down into subsystems to make them easier to understand or design.

🔹 Example of a System

- Security light system:

  - Input: movement detected (e.g. sensor or pressure pad)

  - Process: decision made based on sensor data

  - Output: alarm sounds, lights flash, message sent

🔹 System Diagrams

- Show how a system works using Input → Process → Output structure.

- Help designers understand and plan how a product functions.

🔹 Inputs – Sensors and Switches

- Switches: manually activated (e.g. button or toggle)

- Sensors: automatically detect changes and send signals to the system

⚡ Common Input Components:

- LDR (Light Dependent Resistor): resistance decreases in light, increases in dark

- Thermistors:

  - NTC (Negative Temperature Coefficient): resistance decreases when warm

  - Used in medical thermometers and food safety checks

- Polarity: some components must be connected the correct way (positive/negative)

🔹 Transistors

- Act as a switch or amplifier

- Used to:

  - Switch a large current with a small input (e.g. touch sensor)

  - Amplify a signal (e.g. from a microphone to speaker)

- Key for controlling outputs from small inputs

🔹 Resistors

- Limit current in a circuit

- Used to:

  - Protect LEDs from burning out

  - Control brightness (e.g. in dimmer switches)

  - Pull-up/down resistors help with input signals

- Measured in ohms (Ω)

  - 1kΩ = 1000 ohms

🔹 Outputs – What the System Does

🔸 Buzzer

- Produces sound using a vibrating electromagnet

- Used in alarms, timers, and simple toys

- Cheaper than a speaker, limited sound range

🔸 LEDs (Light Emitting Diodes)

- Emit light when current flows through

- Very low power usage

- Must be connected correctly (only works one way: anode to cathode)

- Must be protected by a resistor (typically 220Ω – 330Ω)

- Common in TVs, monitors, appliances, etc.

- Can shatter if overloaded – handle with care

🔹 Key Terms Summary

- Input: where energy or a signal enters the system (e.g. sensor)

- Process: the system reacts (e.g. decides whether to trigger an alarm)

- Output: the result (e.g. sound, light, movement)

- Polarity: some components only work when connected in the correct direction

- Resistance: slows down the flow of electrical current

- Transistor: switch or amplifier in electronic circuits

# Topic 8 – Programmable Components

🔹 What Are Programmable Components?

- Programmable components are electronic parts that can be given instructions to control the behaviour of a system.

- They are found in many smart products, such as flight systems, washing machines, and toasters.

🔹 Integrated Circuits (ICs) and Microcontrollers

- ICs (Integrated Circuits): tiny electronic circuits sealed in a plastic chip.

- Come in DIL (Dual In-Line) packages with a notch to show orientation (pin 1 is to the left of the notch).

- Microcontrollers (e.g. PICs):

  - Can perform multiple functions

  - Help make products smaller, cheaper, and more efficient

  - Reduce component redundancy – fewer parts needed

  - Use less power and space

🔹 Analogue vs Digital Signals

- Analogue signals: continuous and can have any value (e.g. dimmer switch).

- Digital signals: only on or off, 1 or 0 (e.g. push-button).

- Most PICs can process digital inputs, some can handle analogue inputs too (only on certain pins).

🔹 Flowcharts

- Used to plan programs for programmable components.

- Show the sequence of instructions, decisions, and outputs.

- Common flowchart symbols:

  - Terminator (start/stop)

  - Process box (action or instruction)

  - Decision box (yes/no question)

  - Input/Output box

🔹 Programming a PIC

- Common programming platforms: PICAXE, Genie, Circuit Wizard, Arduino, Raspberry Pi, BBC Micro:bit

- Pros:

  - Makes circuits simpler and smarter

  - Can perform complex functions

- Cons:

  - Can be more expensive

  - Needs PC, software, cables, and skills

🔹 Inputs and Pull-up/down Resistors

- Input devices work better when part of a potential divider circuit.

- Without a clear signal, a switch may be in a 'floating' state (uncertain if on or off).

- Use a pull-up or pull-down resistor to ensure a stable signal is sent to the PIC.

🔹 Timers: Monostable & Astable

- Monostable: gives one pulse of output (e.g. lamp stays on for 10 seconds after movement).

- Astable: produces a repeating pulse (e.g. flashing bike light, car indicators).

- Measured in hertz (Hz):

  - 1 Hz = 1 pulse per second

  - 1 KHz = 1,000 pulses/sec

  - 1 GHz = 1,000,000,000 pulses/sec (used in computers)

🔹 Open vs Closed Loop Systems

- Open loop: runs the same way every time, no feedback (e.g. basic toaster).

- Closed loop: uses feedback to make decisions (e.g. smart oven adjusting temperature).

  - Output can influence the input.

🔹 Counting & Cascading

- Counting: process of keeping track (e.g. number of times a button is pressed).

- 7-segment display: shows numbers (0–9).

  - Can be driven by a decoder IC or programmed directly using a PIC.

- Cascading: one counter triggers another when it reaches a set value (e.g. tens digit increases after units reach 9).

🔹 Key Terms Recap

- IC: small chip with many components

- PIC: programmable chip (microcontroller)

- Digital signal: on/off (1 or 0)

- Analogue signal: variable signal (any value)

- Flowchart: diagram showing instructions

- Monostable: one-time output

- Astable: repeating output

- Open loop: no feedback

- Closed loop: uses feedback

- Pull-up/down resistor: stabilises signals

- Cascading: connecting counters